fullmo Kickdrive Developer's Documentation:
Python Automation Scripts and Remote Control

Kickdrive Version 2.2.5

Date: 2017-06-26

Author: O.Heggelbacher

Copyright 2017 www.fullmo.de , www.fuh-edv.de, All Rights Reserved

CONTENTS

Notes 2

Kickdrive Automation / Python API 3

KickSendMsg("<recipient>:<msgType>:<msgState>[:<msgData>]", "<flags>") 3

KickWaitMsg("<msgWithWildcards>", <timeoutInt>) 4

Example code in Python 4

Kickdrive Remote Control 6

Remote Control Message format <recipient>:<msgType>:<msgState>[:<msgData>] 6

KickModCanOpenGateway - implements some basicCANopen application protocol layer behavior. 6

KickModCanOpenGateway KM_INTERVIEW messages 6

SDOWriteObject 7

SDOReadObject 7

LSSNodeParameters 7

SDODownload 8

SDOAbortDownload 9

SDOUpload 9

SDOAbortUpload 10

ScanIds 10

NmtModuleControl 10

Additional KickModCanOpenGateway KS_INFO messages 11

KickMessageBase 12

Member Function Documentation 13

Member Data Documentation 14

KickMessage – Message Format Overview 17

Detailed Description 17

Static Public Attributes 17

Detailed Description 19

Member Data Documentation 19

KickModCanInterface - The CAN/USB controller class. 32

KickModPosDrive position drive object. 32

Drive Scripts / KM_SCRIPT_CONTROL support: 32

KickModCommChannel. 33

KickModDriveXenax - JennyScience Xenax controller. 33

# Notes

- The following documentation contains parts of the original Kickdrive source code documentation, and may contains references or labels that refer to internal C++ programming details, e.g. C++ class name. These are not relevant for the Kickdrive Automation / Remote Control interfacing.

- This Microsoft Word document was generated using a document generator tool (Doxygen), and may contain minor formatting issues especially when describing parameter lists and string formats used.

KickModAutomation - Automating Kickdrive.

This module allows embedding a Python script that can access Kickdrive functionality via a dedicated API.

# Kickdrive Automation / Python API

This module is used to create and run Python script code that can communicate with other Kickdrive modules (e.g. CANUSB interface, or a CANopen drive object) via KickMessage objects that are converted from/to a simple text string.

Since every GUI panel communicates with such messages, everything that is available in a user dialog (testing drive modes, uploading new firmware, ...) can be automated using a Python script.

For getting started with your own automation script, do the following:

Open a predefined Automation example project, e.g. "demo_automation.kicktpl"

Change to User Level = 2

Go to the "Setup" panel of the Kickdrive Automation module.

Press "Listen" to get a display of the ongoing communication between modules.

Activate "Don't add KM_STATUS messages" to avoid your display being filled with periodical status information from all modules.

Do some manual configuration (e.g. download a new firmware into your CANopen drive)

Have a look at the logged KickMessage objects. Especially watch out for lines that say "GUI ->" - these are the original UI messages that triggered / started the action, e.g. a "KM_DRIVE_DOWNLOAD_START" message to initiate a drive firmware update.

Before executing the first line of the Kickdrive automation script code, a Kickdrive-specific Python module called "Kickdrive" needs to be imported that provides two new functions:

### KickSendMsg("<recipient>:<msgType>:<msgState>[:<msgData>]", "<flags>")

Sends new KickMessage to one or several Kickdrive modules (CanUSB, drives, ..)

##### Parameters:

| <recipient> | The label of the KickModule that should process this event, e.g. "canopen_gateway" for CANopen SDO talks, or "CANopen Drive 1" for controlling the drive like in the drive setup panel. If empty, the message is sent to all modules. |
| --- | --- |
| <msgType> | Kickdrive message type, e.g. KM_COMM_OPEN |
| <msgState> | Kickdrive message state, usually KS_REQUEST for a new request |
| <msgData> | Optional data parameter needed for this request, See the KickMessage.h documentation for a list of available messages and their parameters |
| <flags> | Optional string flags: "w": wait for an answer to this message. NOTE: Not all KickMessages are answered by all slave types. Sometimes a slave just takes a request and runs, without sending the message with state KS_ANSWER back. If in doubt, use the "Listen" function in Kickdrive Automation to find out if a slave actually does or does not respond. "b" : same as w, but the return value is not the whole message, but only a boolean integer that indicates if the request was returned with success (1) or it failed (0) |

##### Returns:

return value is an empty string, if no optional <flags> were specified. Return value is the complete answer message, if "w" was specified for <flags>. Return value is 1 (success) or 0 (failure), if "b" was specified for <flags>.

### KickWaitMsg("<msgWithWildcards>", <timeoutInt>)

Waits for a specific message from one or several Kickdrive modules

##### Parameters:

| msgWithWildcards | empty : wait until timeout "*" : wait for any message non-empty : wait for a matching message, e.g. KickWaitMsg("CAN USB 1:KM_STATUS:KS_INFO:0") –> wait for a status code 0 (idle) from the CANUSB interface answer = KickWaitMsg("*:KM_CANOPEN_EMERGENCY:*", 10000) –> wait for any CANopen emergency, use 10 seconds timeout. <answer> contains all emergency details (see KickMessage.h for the KM_CANOPEN_EMERGENCY syntax) |
| --- | --- |
| timeoutInt | integer with timeout in milliseconds. If not specified, KickWaitMsg waits until a matching message is received. |

##### Returns:

return value is the matching message. If timed out and no matching message was received, return value is and empty string.

### Example code in Python

import Kickdrive

from Kickdrive import *

# start CAN interface

answer = KickSendMsg(":KM_COMM_OPEN:KS_REQUEST", "w")

# after successfully opening the CAN USB port, <answer> contains

# ":KM_COMM_OPEN:KS_ANSWER:"

#  after failing to open the port, <answer> contains

# ":KM_COMM_OPEN:KS_FAIL:"

# set the node id for the "CANopen Drive 1" object in the project tree

KickSendMsg("CANopen Drive 1:KM_SETPROPERTY:KS_REQUEST:NodeId=1", "")

# initiate script download. Use "b" syntax to get a boolean integer to indicate success

success = KickSendMsg("CANopen Drive 1:KM_DRIVE_DOWNLOAD_START:KS_REQUEST:s,D:/temp/demo_links_rechtlauf.pymbin", "b")

if success:

KickSendMsg(":KM_INFOTEXT:KS_INFO:Script download started successfully")

KickSendMsg(":KM_STATUS:KS_INFO:1")

KickWaitMsg("", 1000)

else:

KickSendMsg(":KM_INFOTEXT:KS_INFO:Script download could not be started")

KickSendMsg(":KM_STATUS:KS_INFO:2000")

KickWaitMsg("", 5000)

# Kickdrive Remote Control

The Kickdrive application can be remote controlled. You can use the KickModRemoteControl module in Kickdrive to activate a TCP server port which receives (and responds with) KickMessage objects in plain text format. This is simular to the KickSendMsg and KickWaitMsg mechanism used in the Kickdrive Automation / Python API (see Kickdrive Automation / Python API).

For getting started with Remote Control:

From the Fullmo Kickdrive start screen, open the example project  "Specialized Application" --> "Automation (TCP and Python)"

Find out about the relevant KickMessage objects, as described in the Kickdrive Automation section (see Kickdrive Automation / Python API).

At the Remote Control "Connection" text field, enter SERVER: and the TCP port to use. You can simply accept the default TCP server port displayed, e.g. "SERVER:51111".

Start Remote Control

Now use a TCP client (e.g. the Docklight Scripting development tool) to connect to this server and send text messages in the format specified below.

### Remote Control Message format <recipient>:<msgType>:<msgState>[:<msgData>]

The TCP message format is basically the same format as used for the Kickdrive Automation / KickSendMsg API (see Kickdrive Automation / Python API). Use <LF> (character code 0x0A) for line breaks. To mark the end of a message, use two line breaks in a row <LF><LF>, i.e. an extra empty line.

Example TCP message string: Open any communication device (e.g. CANUSB)

:KM_COMM_OPEN:KS_REQUEST

Example TCP message string: initiate script download for the drive object named 'CANopen Drive'

CANopen Drive:KM_DRIVE_DOWNLOAD_START:KS_REQUEST: s,D:/temp/demo_links_rechtlauf.pymbin

# KickModCanOpenGateway - implements some basicCANopen application protocol layer behavior.

A KickModCanOpenGateway object waits on the slave side for a KickMessage::KM_INTERVIEW request with a CANopen protocol question. It translates it into the CAN telegrams required, sends out the neccessary KickMessage::KM_INTERVIEW requests on the master side and waits for the answer. After completed, it translates this back into the CANopen answer and sends the completed KickMessage::KM_INTERVIEW back on the slave side. KickModCanOpenGateway is far from being a complete CANopen stack, it just does a minimum translation between CANopen and CAN frames and ignores most exceptions, errors and special situations.

## KickModCanOpenGateway KM_INTERVIEW messages

Supported KickMessage::KM_INTERVIEW commands when in "Operating" mode. (See the KickInterview class for general documentation about what a Kickdrive interview is)

### SDOWriteObject

Expedited SDO download for a canunsigned32 ... cansigned8 value question syntax:

<nodeId>,<index>,<subindex>,<dataType>,<value>

<dataType> can be unsigned32,integer32,unsigned16,...,integer8. returned answer syntax:

<nodeId>,<index>,<subindex>[,<error message>, if not successful]

Example interview: "set velocity of can node 2f (hex) to 2000 (decimal). Use 1000 milliseconds timeout and don't retry"

SDOWriteObject:1000:0:2fh,6081h,0h,integer32,2000

Example interview result (everything's fine)

SDOWriteObject:1000:0:2fh,6081h,0h,integer32,2000:2fh,6081h,0h,ok

Example interview result (error)

SDOWriteObject:1000:0:2fh,6081h,0h,integer32,2000:2fh,6081h,0h,error code 6010001h

(In addition, the message state of the KickMessage::KM_INTERVIEW message will be KickMessage::KS_FAIL)

### SDOReadObject

SDO upload request for a canunsigned32 ... cansigned8 value question syntax:

<nodeId>,<index>,<subindex>,<dataType>

returned answer syntax:

<nodeId>,<index>,<subindex>,<dataType>,[<value>|<error message>]

<dataType> can be unsigned32,integer32,unsigned16,...,integer8 and visible_string

Example interview: "get device type of can node 1. Use 500 milliseconds timeout and two retries"

SDOReadObject:500:2:1h,1000h,0h,unsigned32

Example interview result (everything's fine)

SDOReadObject:500:2:1h,1000h,0h,unsigned32:1h,1000h,0h,unsigned32,131474

Example interview result (error, plus KickMessage state is KickMessage::KS_FAIL)

SDOReadObject:500:2:1h,1000h,0h,unsigned32:1h,1000h,0h,unsigned32,error code 6010001h

### LSSNodeParameters

Set device node id and/or baud rate, using LSS Configuration Mode question syntax:

<selectionMode>,<selectionID>,<newNodeId>,<newBaudRate>

##### Parameters:

| <selectionMode> | is a character: g = global, v = vendor id, p = product code, r = revision, s = serial number. |
| --- | --- |
| <selectionID> | the specific vendor Id, product code, ... as an unsigned32. Not used for g = global. |
| <newNodeId> | new ID between 1 and 127, or zero if no change. |
| <newBaudrate> | new CAN baud rate, or zero if no change. |

returned answer syntax:

<selectionMode>,<selectionID>,<newNodeId>,<newBaudRate>,[ok|<error message>]

Example interview: "change node ID of device with serial number 7A651273 (hex) to 2"

LSSNodeParameters:5000:0:s,7A651273h,2,0

example interview answer - everything's fine:

LSSNodeParameters:5000:0:s,7A651273h,2,0:ok

Example interview: global request (changes first node found to ID = 63)

LSSNodeParameters:5000:0:g,0,63,0

Example interview answer - error,

LSSNodeParameters:5000:0:g,0,63,0:error

(plus KickMessage state is KickMessage::KS_FAIL)

### SDODownload

CANopen domain download: Transfering data to the CAN server (the CAN slave node), using either SDO segmented transfer or SDO block transfer question syntax:

<nodeId>,<index>,<subindex>,<flags>,<maxSizePerSession>,<retries>,<data>

##### Parameters:

| <maxSizePerSession> | if <> 0, the transfer is split into individual SDO Download Service sessions of <maxSizePerSession> bytes. |
| --- | --- |
| <retries> | number of retries for an individual block |
| <data> | the download data in QByteArray hex string representation, or the path to the binary file to be used. |
| <flags> | A combination of the following letters |

b : (block) if set, use CANopen block transfer, instead of segmented transfer (segmented is default)

f : (file) if set, the <data> section contains a file path to a raw binary file with the actual data NOTE: every colon (":") of a file path must be replace by ";", since ":" is already the separator for the interview syntax

a : alignment - if <sessionSize> is used, even the very last transfer contains <sessionSize> bytes. a zero byte is used for padding.

n : name field - prepend the binary download data with a 16 bytes name field containing the file name, or 'Untitled' if no file is provided

s : use simplified handshake and only wait for confirmation at each download session end. This is to work around USB latency times and increase data throughput

l : this is a application binary download where the .bin file consists of a number of blocks units with the following structure:

<block number 32bit><flash address 32bit><data size 32bit>

1 .. <data size> bytes

<block crc 32bit>

SDODownload returns immediately after successfully initializing the download. Progress, success or failure is reported using the KickMessage::KM_PROGRESS message.

The download service has been completed when you are receiveing a KM_PROGESS message with integer value 100 (100% complete). If this message is KM_PROGRESS:KS_INFO:100, the download completed successfully. If this messages is KM_PROGRESS:KS_FAIL:100, the download has failed or has been canceled.

returned answer syntax:

<status message>

Example interview

SDODownload:0:0:1,2f50h,2,fhn,0,0,d;/test1.bin

example interview result (everything's fine. Initialized and download in progress)

SDODownload:0:0:1,2f50h,2,fhn,0,0,d;/test1.bin:initialized

Example interview result (error)

SDODownload:0:0:1,2f50h,2,fhn,0,0,d;/test1.bin:error - no data / file not found

### SDOAbortDownload

Abort pending download with error code 08000000h General error Returned answer syntax: (empty string)

Example interview

SDOAbortDownload:0:0:1,2f50h,2

### SDOUpload

CANopen domain upload: Transfering the data to the CAN client (Kickdrive), using a SDO Block Upload service question syntax:

<nodeId>,<index>,<subindex>,<flags>,<blkSize>

##### Parameters:

| <blkSize> | "Number of segments per block" as defined in CANopen. 0 < blksize < 128. Default is 127. |
| --- | --- |
| <flags> | (not used currently) SDOUpload returns immediately after successfully initializing the download. Progress, success or failure is reported using the KickMessage::KM_PROGRESS message. |

The upload service has been completed when you are receiveing a KM_PROGESS message with integer value 100 (100% complete). If this message is KM_PROGRESS:KS_INFO:100, the upload completed successfully. If this messages is KM_PROGRESS:KS_FAIL:100, the upload has failed or has been canceled.

After a successful transfer indication with a KickMessage KM_PROGESS:KS_ANSWER:100, the CANopen gateway object sends a KM_COMM_DATA:KS_INFO:<data> message where <data> is the binary data received. (or an appropriate HEX conversion, if KickMessages are in string format.)

returned answer syntax:

<status message>

Example interview

SDOUpload:0:0:1,2f50h,2,127

example interview result (everything's fine. Initialized and download in progress)

SDOUpload:0:0:1,2f50h,2,127:initialized

Example interview result (error)

SDOUpload:0:0:1,2f50h,2,127:error - illegal blksize

### SDOAbortUpload

Abort pending upload with error code 08000000h General error Returned answer syntax: (empty string) Example interview

SDOAbortUpload:0:0:1,2f50h,2

### ScanIds

Get all node ids available on the bus using SDO Read commands for a default object 0x1000 (Device Type) question syntax:

returned answer syntax:

<id1>,<id2>,<id3>,... ,<idn>

<deviceType1>,<deviceType2>,<deviceType3>, ... <deviceTypeN>,

Example interview:

ScanIds:0:0

Example interview result where three node ids of same device type (01ff0123h) have been detected: 1, 5, 10.

ScanIds:0:0::1,5,10

01ff0123h,01ff0123h,01ff0123h

### NmtModuleControl

Execute a NMT Module Control Command question syntax:

<nodeId>,<CS Byte>

returned answer syntax:

noresponse

Example interview: Reset communication for all nodes

NmtModuleControl:500:0:0,82h

The NMT commands are not committed. Simply waits for the timeout and returns.

NmtModuleControl:500:0:0,82h:noresponse

### Additional KickModCanOpenGateway KS_INFO messages

Additional KickMessage::KS_INFO messages which KickModCanOpenGateway sends automatically when in "Operating" mode

#### KM_CANOPEN_BOOTUP

bootup message detected. Returns the corresponding <nodeID> as integer argument.

#### KM_CANOPEN_EMERGENCY

emergency message detected. Returns a string with the following format:

<Node ID>,<Error Code>,<Error Register>,<manufacturer data As Hex>

# KickMessageBase

A messaging class for communication between KickModule and KickPanel objects.

KickMessageBase is used for message objects that are passed between the various GUI Panels and module objects. The message content is flexible to a maximum degree. It is not intended to define a strict protocol for in-process communication, but rather as a flexible object to communicate anything from a plain string to more complex data structures. Subclasses can define additional message types and override the data parsing from and to strings, using the virtual initData() method.

#### enum KickMessageBase::KickMessageState

KickMessageState.

Each message can have different states that will change during the lifetime of a message object

##### Enumerator:

| KS_INFO | Informational message that does not require action |
| --- | --- |
| KS_REQUEST | Request for information or command. |
| KS_WAITING | (still) waiting for an answer |
| KS_ANSWER | This is a former request that turned into an answer now |
| KS_REJECT | This request is not supported and rejected |
| KS_FAIL | This is a failed request (e.g. an error occurred) |
| KS_TIMEOUT | This is a timed out request that could not be answered |
| KS_CANCEL | request has been canceled |

### Member Function Documentation

#### QVariant KickMessageBase::data () const[inline]

return the data in the original QVariant representation used

dataAsString() and toString() allows conversion to a string

References deleteRecipient(), recipient(), and setRecipient().

Referenced by KickMessageBase().

#### QString KickMessageBase::dataToString () const[virtual]

return the data() member as a string representation

Subclasses may want to override the default conversion, and add more sophisticated conversions. toString() makes use of dataToString().

Referenced by hasOrigin(), and toString().

#### bool KickMessageBase::operator== (const KickMessageBase &  right) const

Messages are compared by ID. Same ID –> operator== will yield 'true'.

Comparison operators are all on an ID basis only. If its the same ID, it's the same thing, even if the status is different. This allows sorted lists and all that

#### void KickMessageBase::setOrigin (const QString &  label)

origin:

same code for the "origin"

Periodic messages of type KS_INFO usually store the originating module's name in "origin". See also setRecipient().

References deleteOrigin(), and pOriginLabel.

Referenced by hasRecipient(), and KickMessageBase().

#### void KickMessageBase::setRecipient (const QString &  labelOrTypeStr)

recipient:

for recipient and origin, only create a string object, if really in use. Otherwise leave empty.

An additional recipient can be specified to address specific receivers on a bus connection where several slaves are listening to one master.

##### Parameters:

| labelOrTypeStr | specifies the recipient's module label. Or, as an alternative, a matching slaveTypeStr |
| --- | --- |

References deleteRecipient(), and pRecipientLabel.

Referenced by data(), and KickMessageBase().

#### QString KickMessageBase::toString (const int  format = 0) const

provide a default 'to string' conversion

see the KickMessageBase string constructor for formats. toString() makes use of dataToString().

##### Note:

for format = 4: If no <origin> is specified, the <recipient> is used instead

References dataToString(), hasOrigin(), origin(), and recipient().

Referenced by hasOrigin().

### Member Data Documentation

#### const int KickMessageBase::KM_APPLICATION_CONTROL = 14[static]

KM_APPLICATION_CONTROL: Application level operations.

Used to perform application-level operations which are also available through the user interface or to deliver application-scope information to different panels. These can be sent by any active KickPanel. KickPanels are automatically connected to the Kickdrive msgIn slot.

KM_APPLICATION_CONTROL has a string argument with the format <cmd>[,<argument>][,<argument2>].. where <cmd> is

"new": create a new project, <argument>provides the template .kicktpl file to use

"open": string argument is the path to open. If empty, a dialog is used

"save": save with current file path/name.

"saveas": string argument for the save path. If empty, regular "save project" is used

"panel": opens the display panel named <argument>. Like clicking on a project tree element.

"exit": quit application

"startprocess": start an external application

"addmodule": add a new project module node

"removemodule": remove project module node(s)

"openurl": open the URL string in the argument in an external standard browser window.

"showhelp": show the help file as defined in the "HelpFile" user setting

"showinfo": show the information / about window as defined in the "InfoUrl" user setting

"resize": resize the main application window to the size passed as string argument with format <width>,<height>

"projectloaded": a KS_INFO message broadcast by the application to all modules/panels, indicating the full project was loaded

"readfile": read file contents

"writefile": create a new file and write contents into it

"appendfile": open/create a file and append contents to it

"readclipboard": get text from clipboard

"writeclipboard": write text into clipboard

#### const int KickMessageBase::KM_AUTOMATION_SNIFFER_MSG = 9[static]

KM_AUTOMATION_SNIFFER_MSG - Informational text with the result of the Listen / Record mode for Automation.

This is a formatted text messages the KickModAutomation module generates after evaluating the KM_SNIFFER_MSG messages picked up in Listen / Record mode.

#### const int KickMessageBase::KM_CUSTOM_MESSAGE_TYPES_BASE_VAL = 100[static]

Use when extending this enum in subclasses.

We can't actually use the value when initializing the value in a base class because both are static - but it is coded in for reference

#### const int KickMessageBase::KM_DROP_DATA = 11[static]

KM_DROP_DATA is used to send drag&drop data to a KickPanel where a default drop action can be offered even by dropping stuff on the project tree tile or somewhere else outside the KickPanel

#### const int KickMessageBase::KM_GETALLPROPERTIES = 4[static]

KM_GETALLPROPERTIES: returns all properties available as a multiline string.

Answer data:

<propName1>=<propValue1>

<propName2>=<propValue2>

...

<propNameN>=<propValueN>

KM_GETALLPROPERTIES messages are handled by the KickModule base class code.

Note: the <propName...> returned is always the full name including additional decorations, e.g. targetPosition [mm]=20.0000 actualPosition [mm]=15.1236

#### const int KickMessageBase::KM_GETPROPERTY = 2[static]

KM_GETPROPERTY: get a property value.

Request data: <propertyName> as string.

Answer data: A string with format <propertyName>=<propertyValue>

KM_GETPROPERTY messages are handled by the KickModule base class code. After being answered, a KM_GETPROPERTY or KM_GETALLPROPERTIES message always has the "origin" property changed to the module that actually contains the property, not the original caller that issued the KM_GETPROPERTY or KM_GETALLPROPERTIES call. This is useful for identifying a property value with the actual KickModule it comes from.

<propertyName> should have the format of a valid XML tag name: no spaces, no brackets, no colons or other extra characters.

As an extension, you can include additional information after a space character, e.g. physical units in square brackets. Everything after the first space character is NOT used to reference / store the property. For example the following two strings can be passed with KM_GETPROPERTY and they will return the same property, "targetPosition":

targetPosition

targetPosition [mm]

targetPosition This is the Target Pos in mm

Note: The data in the message answer (KS_ANSWER status) always returns the short property name as defined in the project XML. So for all three of the above requests the result would be:

targetPosition=20.1278

For getting information about the full name including decorations, use KM_GETALLPROPERTIES

#### const int KickMessageBase::KM_INFOTEXT = 6[static]

KM_INFOTEXT: text messages, e.g. errors or status messages.

KM_INFOTEXT messages should be short status messages about the current module operation, e.g. "Homing", "Position Mode", "Drive Fault". The KickContentDefault panel and other standard panels display these in a colored status line box or a colored push button

#### const int KickMessageBase::KM_PROGRESS = 10[static]

a progress information in % from 0 to 100 for processes that run for a longer time. Finished processes should always return a 100%, and use KS_INFO if the whole operation was successful, and KS_FAIL if there was an error

#### const int KickMessageBase::KM_SETPROPERTY = 3[static]

KM_SETPROPERTY: set a property value.

Request data: <propertyName>=<propertyValue>

Answer data: <propertyName>=<propertyValue>

KM_SETPROPERTY messages are handled by the KickModule base class code. Like in KM_GETPROPERTY, the propertyName may include optional decorations after a space character. E.g the following KM_SETPROPERTY requests do the same:

targetPosition [mm]=20.1278

targetPosition=20.1278

targetPosition [millimeter]=20.1278

Note: the additional physical units are just text decorations for display purposes. They are not used for any kind of calculations within KickModule.

#### const int KickMessageBase::KM_SNIFFER_SET = 7[static]

KM_SET_SNIFFER: 1 - enabled sniffer. 0 - disable.

Enabled/disable sniffer mode where all master out/in messages, plus any GUI signal are translated to a text message for debugging purposes

#### const int KickMessageBase::KM_START = 12[static]

KM_START / KM_STOP: every module should usually support a general start and stop command.

for each module there is many times also a specific command, e.g. "start drive" The general KM_START / KM_STOP message has the same effect, but it never takes a parameter and it is used by the GUI as a general way to bring a module into "started" mode.

##### Note:

Currently only supported by KickModNode, KickModCanInterface and KickModDataLog

#### const int KickMessageBase::KM_STATUS = 5[static]

KM_STATUS: periodical information message with the KickModule's status.

0 - Idle, 1-999 operational, 1000-1999 warning, 2000-2999 error. See also the defined below

#### const int KickMessageBase::KM_WHOAMI = 1[static]

WHOAMI request: returns a multiline string containing

<type>

<label>

#### unsigned long KickMessageBase::msgIdGenerator = 0[static], [protected]

This provides thread-safe unique IDs for all message objects.

Define and initialize the static members.

Referenced by KickMessageBase().

# KickMessage – Message Format Overview

### Detailed Description

A Kickdrive-specific messaging class for communication between KickModule and KickPanel objects.

KickMessage extends the basic KickMessageBase functionality, adding additional message types and overriding toString() and initData().

### Static Public Attributes

KickMessageType enum - message types supported. enum KickMessageState { KS_INFO, KS_REQUEST, KS_WAITING, KS_ANSWER, KS_REJECT, KS_FAIL, KS_TIMEOUT, KS_CANCEL }

static const int KM_INTERVIEW = 101

KM_INTERVIEW: Question + expected answer for an application layer protocol, e.g. CAN or CANOpen.

static const int KM_LOGTEXT = 102

KM_LOGTEXT: additional log messages, e.g. for debugging purposes.

static const int KM_COMM_OPEN = 103

KM_COMM_... communication-related messages for KickModCommChannel, KickModCanInterface and others.

static const int KM_COMM_CLOSE = 104

static const int KM_COMM_STATUS = 105

static const int KM_COMM_SET_BAUD = 106

static const int KM_COMM_DATA = 107

KM_COMM_DATA: communication data to transmit(KS_REQUEST), or received data (KS_INFO)

static const int KM_CAN_FRAME = 108

KM_CAN_FRAME: a CAN frame to be transmitted (KS_REQUEST) or received (KS_INFO)

static const int KM_PLAYERCMD_PLAY = 109

KM_PLAYER_CMD... : player control (start, stop, pause...) e.g. used for the CAN Sender.

static const int KM_PLAYERCMD_STOP = 110

static const int KM_PLAYERCMD_PAUSE = 111

static const int KM_PLAYERCMD_SKIP = 112

static const int KM_PLAYERCMD_BACK = 113

static const int KM_PLAYER_STATUS = 114

static const int KM_DRIVE_START = 115

KM_DRIVE_START and other messages the KickModPosDrive accepts from a GUI panel.

static const int KM_DRIVE_STOP = 116

static const int KM_DRIVE_STATUS = 117

static const int KM_SET_PROFILE_MODE = 118

static const int KM_GET_ACTUAL_VELOCITY = 119

static const int KM_GET_ACTUAL_POSITION = 120

static const int KM_DRIVE_NEW_ID = 121

KM_DRIVE_NEW_ID: set a new (CAN) ID. Only when drive stopped.

static const int KM_DRIVE_NEW_COMM_PARAMETERS = 122

static const int KM_DRIVE_RESET = 123

KM_DRIVE_RESET: Perform different reset operations.

static const int KM_DRIVE_GO_TO_TARGET = 124

KM_DRIVE_GO_TO_TARGET: starts positioning.

static const int KM_DRIVE_GO_TO_EXTENDED = 125

KM_DRIVE_GO_TO_EXTENDED: extended positioning command where target postion and all additional parameters are.

static const int KM_DRIVE_START_HOMING = 126

KM_DRIVE_START_HOMING: starts homing procedure or sets new actual position to zero (homing mode 35)

static const int KM_DRIVE_DOWNLOAD_START = 127

KM_DRIVE_DOWNLOAD_START: download and run new drive application firmware or embedded pythons script.

static const int KM_OBJLIST_WRITE = 128

KM_OBJLIST_WRITE: writes a list of parameters, passed as a string.

static const int KM_OBJLIST_READ = 129

KM_OBJLIST_READ: read new object values for a list with the same format as for KM_OBJLIST_WRITE.

static const int KM_OBJLIST_VERIFY = 130

KM_OBJLIST_VERIFY: read values and compare against the provided values.

static const int KM_DIALOG = 131

KM_DIALOG is used to provide dialog elements for automation applications (e.g. a Python script)

static const int KM_CANOPEN_BOOTUP = 134

informational messages the canOpen gateway sends

static const int KM_CANOPEN_EMERGENCY = 135

static const int KM_DAQ_DATA = 136

KM_DAQ_DATA - Data Acquisition.

static const int KM_DAQ_CONFIG = 137

KM_DAQ_CONFIG - Setup Data Acquisition.

static const int KM_DAQ_SHARED_IDS_LIST = 138

KM_DAQ_SHARED_IDS_LIST - list of all known shared data objects and their corresponding shared ids.

static const int KM_DAQ_CONTROL = 139

KM_DAQ_CONTROL - Controls data acquisition using a set of codes representing commands.

static const int KM_SCRIPT_CONTROL = 140

KM_SCRIPT_CONTROL - Controls run/stop states of python scripts.

static const int KM_SCRIPT_MSG = 141

KM_SCRIPT_MSG - Information regarding script state and runtime errors.

static const int KM_MASTER_OUT = 142

KM_MASTER_OUT - take the string argument, convert it into a KickMessage and post it on the module's master side.

static const int KM_DOCKLIGHT = 143

KM_DOCKLIGHT - Used for communication with the Docklight Automation DLL API.

### Detailed Description

A messaging class for communication between KickModule and KickPanel objects.

KickMessage is used for message objects that are passed between the various GUI Panels and managers. The message content is flexible to a maximum degree. It is not intended to define a strict protocol for in-process communication, but rather a flexible object to communicate anything from a plain string to more complex data structures

### Member Data Documentation

#### const int KickMessage::KM_CAN_FRAME = 108[static]

KM_CAN_FRAME: a CAN frame to be transmitted (KS_REQUEST) or received (KS_INFO)

The CAN interface accepts KM_CAN_FRAME / KS_REQUEST and transmits them. The data format is a QByteArray with size 16 that is a 1:1 mapping of the KickCanFrame data structure (we need to use QByteArray as a valid subtype for QVariant. Cannot do this with a custom struct.) the CAN interface returns this message with state KS_ANSWER, if successful, or KS_FAIL if failed to transmit The CAN interface will post all CAN frames received as a KM_CAN_FRAME / KS_INFO automatically. KM_CAN_FRAME can be represented as readable text in the following format:

<msg ID>,<DLC / data size>,<data Hex (up to 8 bytes)>,<timestamp (optional)>

Examples:

7e5h,2,04 00

601h,8,a0 08 10 02 21 00 00 00,564.17

CAN 2.0 B extended frames are preceeded by the keyword 'ext ', e.g.

ext 21386h,8,f1 f2 f3 f4 f5 f6 f7 ff,9999

Referenced by KickModCanOpenGateway::createWorker(), dataToString(), initConversionMaps(), EthernetStates::Root::isStartMsg(), CanUsbStates::Root::isStartMsg(), KickModule::newSlaveOut(), CanOpenGateway::Downloading::nextDownloadSession(), CanOpenGateway::Operating::processCanEvents(), CanUsbStates::Open::readNewFrames(), CanUsbStates::Playing::receiveFrameProcessing(), and CanOpenGateway::Operating::sendAsGatewayMaster().

#### const int KickMessage::KM_CANCEL_INTERVIEWS = 144[static]

KM_CANCEL_INTERVIEWS: all pending interviews are canceled and return with KS_CANCEL state.

If a KM_CANCEL_INTERVIEWS message has the <origin> property set, only the interviews with the same origin are canceled.

Referenced by KickModCanOpenGateway::createWorker(), initConversionMaps(), KickNodeStates::NodeRoot::processBasicRequests(), KickNodeStates::DriveRoot::processRootCommands(), and KickModule::queueInSignal().

#### const int KickMessage::KM_CANOPEN_BOOTUP = 134[static]

informational messages the canOpen gateway sends

bootup message detected for <nodeID> passed as integer argument

Referenced by initConversionMaps(), CanOpenGateway::Operating::processCanEvents(), and KickNodeStates::DriveRoot::processRootCommands().

#### const int KickMessage::KM_CANOPEN_EMERGENCY = 135[static]

emergency message detected. Returns a string with

<Node ID>,<Error Code>,<Error Register>,<manufacturer data As Hex>

Referenced by initConversionMaps(), CanOpenGateway::Operating::processCanEvents(), KickNodeStates::MgrRoot::processRootCommands(), and KickNodeStates::DriveRoot::processRootCommands().

#### const int KickMessage::KM_COMM_CLOSE = 104[static]

close CAN interface / communication device

Referenced by KickModRemoteControl::createWorker(), initConversionMaps(), EthernetStates::Root::isStartMsg(), JoystickInterfaceStates::Root::isStartMsg(), CommChannelStates::CommRoot::isStartMsg(), CanUsbStates::Root::isStartMsg(), RemoteControl::Networking::processIncomingNetworkMsg(), and KickNodeStates::DriveRoot::processRootCommands().

#### const int KickMessage::KM_COMM_DATA = 107[static]

KM_COMM_DATA: communication data to transmit(KS_REQUEST), or received data (KS_INFO)

KickModCommChannel accepts KM_COMM_DATA requests and transmits them, both QByteArray data in binary form or QString text data. KickModDocklight uses KM_COMM_DATA messages to make incoming communication data and Receive Sequence comments accessible.

Referenced by KickModDocklight::createWorker(), dataToString(), initConversionMaps(), CanOpenGateway::Downloading::nextDownloadSession(), and CommChannelStates::Connecting::setRuntimeInfoEx().

#### const int KickMessage::KM_COMM_OPEN = 103[static]

KM_COMM_... communication-related messages for KickModCommChannel, KickModCanInterface and others.

open CAN interface / communication device

Referenced by KickModDataLog::createWorker(), KickModRemoteControl::createWorker(), initConversionMaps(), EthernetStates::Root::isStartMsg(), JoystickInterfaceStates::Root::isStartMsg(), CommChannelStates::CommRoot::isStartMsg(), CanUsbStates::Root::isStartMsg(), KickNodeStates::NodeRoot::objListProcessAnswers(), and RemoteControl::Networking::processIncomingNetworkMsg().

#### const int KickMessage::KM_COMM_SET_BAUD = 106[static]

sets the baudrate. Identical to setting the CANINTERFACE_SETTINGS_BAUD property, but can be tunneled through the canOpen gateway. Only works in IDLE mode

Referenced by initConversionMaps(), CanUsbStates::Root::isStartMsg(), KickNodeStates::DriveRoot::processRootCommands(), and CanUsbStates::Playing::receiveFrameProcessing().

#### const int KickMessage::KM_COMM_STATUS = 105[static]

info message, returns integer bit value: 1 - TX data, 2 - RX data, 4 - ERROR, 8 - BUS ERROR

Referenced by KickModCanOpenGateway::createWorker(), initConversionMaps(), CanUsbStates::Root::isStartMsg(), KickModule::newSlaveOut(), and Automation::Root::reportMasterIn().

#### const int KickMessage::KM_DAQ_CONFIG = 137[static]

KM_DAQ_CONFIG - Setup Data Acquisition.

This message represents either a request to monitor data OR an answer to such a request.

The data for a KS_REQUEST looks like:

<dataObjId>,<name>,<dataType>,<method>,<interval>

When confirmed as KS_ANSWER from the data acquiring module (e.g. KickModCanOpenGateway), the data is:

<dataObjId>,<name>,<dataType>,<method>,<interval>,<scalingExpression>,<status:1=success,0=failed>,-1

The KickModDataLog module receives and forwards this to its KickPanel ui elements, and tags on the id of the shared data container:

<dataObjId>,<name>,<dataType>,<method>,<interval>,<scalingExpression>,<status:1=success,0=failed>,<sharedDataId>

Referenced by KickModDataLog::createWorker(), KickModCanOpenGateway::createWorker(), KickContentPlot::getNewPlotDocsXml(), initConversionMaps(), CanOpenGateway::Downloading::nextDownloadSession(), DataLogStates::FileMode::processFileMessages(), and DataLogStates::LiveRunning::processLiveMessages().

#### const int KickMessage::KM_DAQ_CONTROL = 139[static]

KM_DAQ_CONTROL - Controls data acquisition using a set of codes representing commands.

Data is a comma-delimited list of values.

First argument is a code representing a Data Acquisition related command. The different codes and their arguments are documented in KickModDataLog::DaqControlCodes enum.

This replaces the obsolete types: KM_DAQ_LOAD_FILE / KM_DAQ_CLEAR / KM_DAQ_RECORD_STOP / KM_DAQ_RECORD_START.

Referenced by KickModDataLog::createWorker(), KickModDataLog::getDaqControlCode(), initConversionMaps(), DataLogStates::FileMode::processFileMessages(), and DataLogStates::LiveRunning::processLiveMessages().

#### const int KickMessage::KM_DAQ_DATA = 136[static]

KM_DAQ_DATA - Data Acquisition.

Data Format:

<dataObjId>,<date/time>,

Several measurements can be transmitted in one KM_DAQ_DATA message. Use a line break <LF> character to separate the different lines of data.

The <dataObjId> format is as defined in KickBabelFish::dataObjectId(..):

"<nodeId>.<indexHex>h.<subindex>h"

with fixed length numbers, example: "003.107fh.02h"

Referenced by initConversionMaps(), CanOpenGateway::Downloading::nextDownloadSession(), and DataLogStates::LiveRunning::processLiveMessages().

#### const int KickMessage::KM_DAQ_SHARED_IDS_LIST = 138[static]

KM_DAQ_SHARED_IDS_LIST - list of all known shared data objects and their corresponding shared ids.

This is sent periodically by the data acquisition module, but can also be requested with an empty KS_REQUEST message of this type.

Data is a multiline string in the following format:

<data_object_id>,<shared_id>

<data_object_id>,<shared_id>

...

Referenced by KickContentPlot::getNewPlotDocsXml(), initConversionMaps(), DataLogStates::FileMode::processFileMessages(), and DataLogStates::Root::sendSharedIdsMessage().

#### const int KickMessage::KM_DIALOG = 131[static]

KM_DIALOG is used to provide dialog elements for automation applications (e.g. a Python script)

Message boxes - use the following string argument:

<type>,<message>,<default>

<type> = "s": ask for string input value. Return message state is KS_ANSWER, if ok, KS_CANCEL if user canceled

<type> = "i": information box with OK / Cancel. Return message state is KS_ANSWER, if ok, KS_CANCEL if user canceled

<type> = "w": same as "i", but use warning, not informational, and Cancel is default

File dialogs - use the following string argument

<type>,<caption>,<dir>,<filter>

<type>: "fo" for 'Open File', "fs" for 'Save File'

: The dialog's caption is set to caption. If caption is not specified then a default caption will be used.

<dir>: The file dialog's working directory will be set to dir. If dir includes a file name, the file will be selected. If dir is empty, the last known path from a project open is used. If dir is "%%WorkingDir%%", the path is the current Temp location where a .kickzip project has been extracted to. Or - if you are using a .kickpro file - the regular project path.

<filter>: Only files that match the given filter are shown. If you want multiple filters, separate them with ';;', for example:

"Images (*.png *.xpm *.jpg);;Text files (*.txt);;XML files (*.xml)"

Referenced by Kickdrive::appControlIn(), initConversionMaps(), and KickModule::newSlaveOut().

#### const int KickMessage::KM_DOCKLIGHT = 143[static]

KM_DOCKLIGHT - Used for communication with the Docklight Automation DLL API.

The message data syntax is based on the syntax and functionality of the DocklightDL object.

The data for a KS_REQUEST looks like: <Docklight api="" call="" with="" arguments>="">

The KS_ANSWER would be: <Docklight api="" call="" with="" arguments>="">:<return value>="">

The KS_FAIL would include the error information from the DL object: <Docklight api="" call="" with="" arguments>="">:<number from DlGetDocklightErrorNo()>,<description from DlGetDocklightErrorDescription()>

Referenced by KickModDocklight::createWorker(), and initConversionMaps().

#### const int KickMessage::KM_DRIVE_DOWNLOAD_START = 127[static]

KM_DRIVE_DOWNLOAD_START: download and run new drive application firmware or embedded pythons script.

String parameter to specifiy the download:

<downloadType>,<downloadBinaryPath>

<downloadType> = s: stops the running script, loads the script from the raw binary file path in the argument, and starts the new script.

<downloadType> = f: switch to bootloader, download a new application firmware from the binary specified, and restart the application code.

<downloadType> = b: same as 'f', but use CANopen block transfer mode, if available

<downloadType> = c: same as 'f', but use a fullmo usb2drive downlad via USB removable media (SD card) and usb2drive's built in firmware updater. Application for this is e.g. ebmpapst K4 drive.

Use KM_DRIVE_STOP to cancel any running download and go back to idle

Referenced by initConversionMaps(), KickContentPythonEditor::on_actionPlay_triggered(), KickNodeStates::DriveRoot::processRootCommands(), and KickNodeStates::DriveRoot::targetPosition().

#### const int KickMessage::KM_DRIVE_GO_TO_EXTENDED = 125[static]

KM_DRIVE_GO_TO_EXTENDED: extended positioning command where target postion and all additional parameters are.

extended positioning command where target postion and all additional parameters are specified through the following, comma-separated argument list:

<targetPos>,<flags>,<velocity>,<acceleration>,<deceleration>

<targetPos>: new end position (float)

<flags>:

't' - require that 'target reached' status bit is set before setting a position. If 'target reached' is not set, this message returns with "KS_FAIL" state.

'i' - monitor 'target reached' during positioning and report back, when target is reached with another KM_DRIVE_GO_TO_EXTENDED message with status "KS_INFO" and text "target reached".

<velocity>,<acceleration>,<deceleration>:

optional integer ramp parameters. Only used if > 0.

KM_DRIVE_GO_TO_EXTENDED always reads the drive's status word as a first step and checks if the drive is still busy. If flag 't' (target reached) is not set, the message is returned with KS_CANCEL status, and the new target is not set.

After accepting the positioning command, the drive returns KM_DRIVE_GO_TO_EXTENDED with status KS_ANSWER

If the flag 'i' is set, KM_DRIVE_GO_TO_EXTENDED is answered a second time with status 'KS_INFO' and data string "target reached" after the drive reached the target position.

Referenced by KickModTestCycle::createWorker(), initConversionMaps(), KickNodeStates::DriveRoot::processRootCommands(), and DriveXenaxStates::Init::tick().

#### const int KickMessage::KM_DRIVE_GO_TO_TARGET = 124[static]

KM_DRIVE_GO_TO_TARGET: starts positioning.

Optional int parameter to specify position mode:

1 : new set point & start

2 : new set point & halt

3 : halt

4 : release after halt

target velocity and limits are set before as drive properties (see KM_SETPROPERTY)

Referenced by initConversionMaps(), and KickNodeStates::DriveRoot::processRootCommands().

#### const int KickMessage::KM_DRIVE_NEW_COMM_PARAMETERS = 122[static]

set a new baud rate. Only when drive stopped. Argument is string with format <newBaudRate>[,<deviceSerialNo>]

Referenced by initConversionMaps(), and KickNodeStates::DriveRoot::targetPosition().

#### const int KickMessage::KM_DRIVE_NEW_ID = 121[static]

KM_DRIVE_NEW_ID: set a new (CAN) ID. Only when drive stopped.

Argument is string with format <newNodeId>[,<deviceSerialNo>]

if the optional <deviceSerialNo> is missing or zero, a LSS Switch Global is used and every CAN node in the CANopen net will respond to the Node ID change

If <deviceSerialNo> is > 0, a LSS Switch Selective for this specific device serial no. is used

If <deviceSerialNo> is "read", the drive module reads out its serial no. first, then it changes the node ID using this serial no.

Referenced by initConversionMaps(), and KickNodeStates::DriveRoot::targetPosition().

#### const int KickMessage::KM_DRIVE_RESET = 123[static]

KM_DRIVE_RESET: Perform different reset operations.

int parameter to specifiy reset mode:

1 : restart application firmware

2 : reset parameters to factory settings

Referenced by initConversionMaps(), and KickNodeStates::DriveRoot::targetPosition().

#### const int KickMessage::KM_DRIVE_START = 115[static]

KM_DRIVE_START and other messages the KickModPosDrive accepts from a GUI panel.

put a drive into operational mode

Referenced by KickModTestCycle::createWorker(), initConversionMaps(), DriveXenaxStates::XenaxRoot::isStartMsg(), and KickNodeStates::NodeRoot::isStartMsg().

#### const int KickMessage::KM_DRIVE_START_HOMING = 126[static]

KM_DRIVE_START_HOMING: starts homing procedure or sets new actual position to zero (homing mode 35)

Int parameter to specify homing mode:

1 - 34: start homing procedure 1..34

35: set new home position (set actual position to zero)

Referenced by KickModTestCycle::createWorker(), initConversionMaps(), and KickNodeStates::DriveRoot::processRootCommands().

#### const int KickMessage::KM_DRIVE_STATUS = 117[static]

periodic KS_INFO sent. Returned status is in Hex 16 bit format

Referenced by initConversionMaps(), and KickNodeStates::DriveRoot::processRootCommands().

#### const int KickMessage::KM_DRIVE_STOP = 116[static]

stop a drive

Referenced by KickContentPythonEditor::actionDownloadTriggered(), Kickdrive::appControl(), KickModTestCycle::createWorker(), initConversionMaps(), DriveXenaxStates::XenaxRoot::isStartMsg(), and KickNodeStates::NodeRoot::isStartMsg().

#### const int KickMessage::KM_GET_ACTUAL_POSITION = 120[static]

returns the actual drive position as a scaled float value

Referenced by initConversionMaps(), KickNodeStates::DriveRoot::processRootCommands(), and DriveXenaxStates::XenaxRoot::targetPosition().

#### const int KickMessage::KM_GET_ACTUAL_VELOCITY = 119[static]

returns the actual velocity as long int

Referenced by initConversionMaps(), and KickNodeStates::DriveRoot::processRootCommands().

#### const int KickMessage::KM_INTERVIEW = 101[static]

KM_INTERVIEW: Question + expected answer for an application layer protocol, e.g. CAN or CANOpen.

An interview request consists of a question (a protocol message to sent to a device, like "what is the current position?") and a definition for an expected answer, which can also include wildcards for expected return values.

Parameter for a KM_INTERVIEW is a string with the following syntax:

<type>:<timeout>:<retry>:<questionString>:<expectedAnswer>

<interviewType> - e.g. "CAN", "SDOWriteObject", depending on what the KickModule object supports

<timeout> - 0 = no timeout or >0 = timeout in milliseconds

<retry> - what to do if request timed out: 0 : return with failure, greater than 0 : retry <x> times. smaller than 0 : retry infinitively.

<questionString> = the protocol request

<expectedAnswer> = optional. the expected answer (may include wildcards like * or ?). If <expectedAnswer> is "noresponse", the interview simply waits until timed out and returns success

After sending the protocol request and receiving a matching answer, <expectedAnswer> is replaced by the actual data and the message is returned to sender with the new state KS_ANSWER.

Example interview:

CAN:1000:1:7e5h,8,17 01 00 00 00 00 00 00:7e4h,8,17*

Example interview after completion and sending back to origin:

CAN:1000:1:7e5h,8,17 01 00 00 00 00 00 00:7e4h,8,17 00 00 00 00 00 00 00

An interview can also contain several questions in several lines of text that are asked one by one. If one question fails / times out, the whole interview fails, unless the first interview line says "OnErrorContinue:".

Example interview:

OnErrorContinue:

SDOWriteObject:1000:2:127,6085h,0h,integer32,1000

SDOWriteObject:1000:2:127,4000h,1h,unsigned8,0

SDOWriteObject:1000:2:127,4000h,4h,unsigned8,5

The return result will then contain the amount of failed / succeeded requests:

OnErrorContinue:2:1

SDOWriteObject:1000:2:127,6085h,0h,integer32,1000,ok

SDOWriteObject:1000:2:127,4000h,1h,unsigned8,0,error

SDOWriteObject:1000:2:127,4000h,4h,unsigned8,5,ok

Another example interview from the KickModPosDrive class

this is how the drive is initialized with one single KM_INTERVIEW:

NmtModuleControl:200:0:1,82h

SDOWriteObject:1000:10:1,6040h,0h,unsigned16,128

SDOWriteObject:1000:0:1,3500h,2h,integer8,0:noresponse

SDOReadObject:1000:2:1,608fh,1h,unsigned32

SDOReadObject:1000:2:1,608fh,2h,unsigned32

SDOWriteObject:1000:2:1,6060h,0h,integer8,3

SDOWriteObject:1000:2:1,60ffh,0h,integer32,0

SDOWriteObject:1000:10:1,6040h,0h,unsigned16,6

SDOReadObject:1000:2:1,6040h,0h,unsigned16:1,6040h,0h,unsigned16,6

SDOWriteObject:1000:10:1,6040h,0h,unsigned16,7

SDOReadObject:1000:2:1,6040h,0h,unsigned16:1,6040h,0h,unsigned16,7

The receiver and processor of this KM_INTERVIEW message is the KickModCanopenGateway class, which understands the SDO and Nmt tokens and will return the result to the KickModPosDrive class after initialization is complete.

KM_INTERVIEW messages are handled by the KickModule base class code. See the KickInterview class for additional documentation.

Referenced by KickModCanOpenGateway::createWorker(), KickModule::getLabel(), initConversionMaps(), CanOpenGateway::Downloading::nextDownloadSession(), KickNodeStates::NodeRoot::objListProcessAnswers(), KickNodeStates::NodeRoot::objListProcessRequests(), KickNodeStates::MgrRoot::processRootCommands(), KickNodeStates::DriveRoot::processRootCommands(), KickModule::queueInSignal(), and CanOpenGateway::Operating::sendAsGatewayMaster().

#### const int KickMessage::KM_LOGTEXT = 102[static]

KM_LOGTEXT: additional log messages, e.g. for debugging purposes.

KM_LOGTEXT messages can contain larger pieces of timestamped logging information. The KickContentDefault panel displays these messages in an extra text edit widget.

Referenced by CommChannelStates::CommRoot::collectLog(), DriveXenaxStates::XenaxRoot::collectLog(), KickModDocklight::createWorker(), KickModTestCycle::createWorker(), initConversionMaps(), and KickNodeStates::NodeRoot::objListProcessAnswers().

#### const int KickMessage::KM_MASTER_OUT = 142[static]

KM_MASTER_OUT - take the string argument, convert it into a KickMessage and post it on the module's master side.

This is the basic KickModule command that can be used by KickPanel objects to post stuff to the "outside world" beyond the KickModule the panel is attached to. The string format for the message is "<recipient>:<msgType>:<msgState>:<dataPartAsString>". The origin is set to the origin of the KM_MASTER_OUT message.

Referenced by initConversionMaps(), KickNodeStates::NodeRoot::processBasicRequests(), and KickQmlInterface::qmlMessageStrOut().

#### const int KickMessage::KM_OBJLIST_READ = 129[static]

KM_OBJLIST_READ: read new object values for a list with the same format as for KM_OBJLIST_WRITE.

The actual value is always returned in decimal format.

Referenced by KickObjEditModel::buildReadWriteRequest(), initConversionMaps(), KickObjEditModel::objectUpdateIn(), KickNodeStates::NodeRoot::objListAddRequest(), KickObjEditModel::processPendingReadWriteRequest(), KickObjEditModel::processReadWriteAnswer(), and KickObjEditModel::setGlobalUpdateInterval().

#### const int KickMessage::KM_OBJLIST_VERIFY = 130[static]

KM_OBJLIST_VERIFY: read values and compare against the provided values.

Uses the same list format as KM_OBJLIST_WRITE, but reads back values instead of setting them and compares them against the provided (assumed) values. The returned result contains the same values as provided, but the status / error column will indicate the result of the comparison, resp. the different SDO value that was actually read from the device.

Referenced by KickObjEditModel::buildReadWriteRequest(), initConversionMaps(), KickNodeStates::NodeRoot::objListAddRequest(), KickNodeStates::NodeRoot::objListProcessAnswers(), KickNodeStates::NodeRoot::objListProcessRequests(), and KickObjEditModel::processReadWriteAnswer().

#### const int KickMessage::KM_OBJLIST_WRITE = 128[static]

KM_OBJLIST_WRITE: writes a list of parameters, passed as a string.

Syntax 1 - Data Object ID format:

<dataObjectId 1>,<valueType 1>,<value 1> # <optional comment, e.g. the attribute's real world name>

<dataObjectId 2>,<valueType 2>,<value 2>

...

<dataObjectId n>,<valueType n>,<value 3>

<dataObjId> format as defined in KickBabelFish::dataObjectId(..):

<nodeId>.<indexHex>h.<subindex>h

with fixed length numbers, e.g.

003.107fh.02h

For

you can use decimal values, or hex values in the format "0x1af" or "1afh", or even bit values in the format "0b10110111".

Classic format (no data object ids, use the nodeID from the application module property "nodeId":

<attributes 1>,<index 1>,<subindex 1>,<valueType 1>,<value 1> # <optional comment>

<attributes 2>,<index 2>,<subindex 2>,<valueType 2>,<value 2>

...

<attributes 3>,<index 3>,<subindex 3>,<valueType 3>,<value 3>

<attribute>: accessibility attributes like "rw", "ro", "wo", "const"

The KickModNode class that implements KM_OBJLIST_WRITE behavior also supports additional text macros instead of a object definition line:

"StoreCmd" : send command to make the object values permanent or

"ResetCmd" : reset to factor settings.

'StoreCmd' and 'ResetCmd' are executed in the order they appear in the object list.

Optional macro than can be used at the start of a object list:

"SilentReadWrite": KickModNode will not change its runtime status or report about success/failure of the object write using KM_STATUS / KM_INFOTEXT.

See KickModNode header for details

Referenced by KickContentPythonEditor::actionDownloadTriggered(), KickContentNodeConfig::actionRead(), KickObjEditModel::buildReadWriteRequest(), KickObjEditModel::getObjectListWriteString(), initConversionMaps(), KickNodeStates::NodeRoot::objListAddRequest(), KickNodeStates::NodeRoot::objListProcessAnswers(), KickNodeStates::NodeRoot::objListProcessRequests(), KickObjEditModel::processPendingReadWriteRequest(), KickObjEditModel::processReadWriteAnswer(), KickNodeStates::DriveRoot::processRootCommands(), and KickNodeStates::NodeRoot::setAccessKey().

#### const int KickMessage::KM_PLAYER_STATUS = 114[static]

info message from the player: reports the current line no. processed. -1 = program ended

Referenced by initConversionMaps(), CanUsbStates::Root::isStartMsg(), CanUsbStates::Playing::receiveFrameProcessing(), and CanUsbStates::Open::setRuntimeInfoCanError().

#### const int KickMessage::KM_PLAYERCMD_PLAY = 109[static]

KM_PLAYER_CMD... : player control (start, stop, pause...) e.g. used for the CAN Sender.

KM_PLAYERCMD_PLAY arguments are either:

an integer no. with the CAN sender program no. from 0 to 9

a string "<programNo>,<initCode>", where <programNo> is the integer program no., and <initCode> is some macro code that should be executed before the first program line, e.g. for initializing some variables.

Referenced by initConversionMaps(), and CanUsbStates::Root::isStartMsg().

#### const int KickMessage::KM_SCRIPT_CONTROL = 140[static]

KM_SCRIPT_CONTROL - Controls run/stop states of python scripts.

Data is a string with the following argument format:

<1=Start|0=Stop>[,<scriptCode>]

Example:

"1" - Start script, using the default script (e.g. as defined by the "ScriptFileName" property in KickModAutomation).

"0" - Stop script

Optional arguments are supported for KickModAutomation:

<scriptCode> contains custom Python script code to be executed instead of the default script.

If the <scriptCode> starts with the keyword "append:", the default script is used, and the part after "append:" is appended to it.

Example:

1,append:myFunction(3)

loads the default script and executes the function call myFunction(3). In this case the default script usually consists of several function definitions without a main().

##### Note:

KM_SCRIPT_CONTROL replaces the obsolete KM_AUTOMATION_RUN_SCRIPT / KM_AUTOMATION_STOP

Referenced by KickContentPythonEditor::actionDownloadTriggered(), initConversionMaps(), KickContentPythonEditor::on_actionPlay_triggered(), KickPanel::processAnchorClicked(), Automation::Root::processBasicCommands(), and KickNodeStates::DriveRoot::processRootCommands().

#### const int KickMessage::KM_SCRIPT_MSG = 141[static]

KM_SCRIPT_MSG - Information regarding script state and runtime errors.

This message represents runtime data from a running python script. The data can be an status/error information reported by the script module (e.g. KickModAutomation), or redirected Python StdOut / StdErr streams.

<line_number>,<runtime_info>

<line_number>: Related line in python code. 0 when line number is unknown or message is not line-specific.

<runtime_info>: Actual data / message to pass on. Additional tokens "StdOut:" and "StdErr:" are used at the beginning of messages that are actual Python StdOut / StdErr texts.

Referenced by initConversionMaps(), Automation::Root::processBasicCommands(), and KickNodeStates::DriveRoot::processRootCommands().

#### const int KickMessage::KM_SET_PROFILE_MODE = 118[static]

put in profile velocity mode (integer argment = 3), position mode (1) or homing mode (6)

Referenced by KickModTestCycle::createWorker(), initConversionMaps(), and KickNodeStates::DriveRoot::processRootCommands().

# KickModCanInterface - The CAN/USB controller class.

KickModCanInterface controls access to one CANUSB device and deals with all CAN-specific operations: Sending frames, receiving frames. It also includes a "Player" functionality that allows to send a predefined set of CAN frames.

KickModCanInterface sends and receives can frames as KickMessage::KM_CAN_FRAME message type. KickModCanInterface answers KickMessage::KM_INTERVIEW messages for protocol specifier "CAN": Example for a completed KickMessage::KM_INTERVIEW data set:

(<interviewType>:<timeout>:<retry>:<questionString>:<expectedAnswer>:actual answer) CAN:1000:1:7e5h,8,17 01 00 00 00 00 00 00:7e4h,8,17*:7e4h,8,17 00 00 00 00 00 00 00

# KickModPosDrive position drive object.

Position drive object with functionality for basic operation (velocity, position mode, homing), and node firmware update & management. Supports fullmo / ebmpapst VDC/K5/K4 drives, but should also work with standard CANopen drives that implement state machine control via objects / SDO according to CANopen DS 402.

### Drive Scripts / KM_SCRIPT_CONTROL support:

The KickModPosDrive module supports the KM_SCRIPT_CONTROL script control message. Standard script control message syntax (in text representation):

KM_SCRIPT_CONTROL:KS_REQUEST:1

KM_SCRIPT_CONTROL:KS_REQUEST:0

to switch the script interpreter inside the drive on and off.

Extended syntax:

KM_SCRIPT_CONTROL:KS_REQUEST:1,<myPythonCode>

compiles the script code provided in <myPythonCode>, downloads it to the drive and runs it.

Example:

KM_SCRIPT_CONTROL:KS_REQUEST:1,import targetlib

while (1):

targetlib.SendEmcyMsg(0xDC)

In this case, the drive will perform the following actions:

save the script under the current drive script name, as open in the Drive Script editor panel

preprocess and compile the script

run the script

# KickModCommChannel.

Basic TCP communication channel for both ASCII and binary protocols. Use the following properties:

##### Parameters:

| CommChannel | The TCP client address in format 192.168.2.50:10001 |
| --- | --- |
| Mode | 0 - ASCII mode, telegram based. CR is used as a telegram separator, any occuring LF or other control characters are stripped from the data. 1 - Binary mode. No separator, just pass any received TCP frame without splitting into telegrams. |
| ManualCommand | If this property is changed, its new value is used as a new ASCII telegram (or as a HEX-encoded binary telegram), and sent out on the TCP connection. |
| ManualCommandAnswer | is updated with the next telegram that is received after a "ManualCommand" has been sent out. |

# KickModDriveXenax - JennyScience Xenax controller.

This is a prototype for representing a JennyScience Xenax controller. This first version allows only very basic control, using basic protocol implementations from the FullmoCommLib C/C++ library, resp. an implmenetation similar to KickModCommChannel.