docklight_demo_remotecontrol.ptp / docklight_demo_remotecontrol.pts

Date: 2020-07-02

Author: Heggelbacher

This is a small demonstration of remote-controlling Kickdrive via a TCP server server socket. The client can connect as a standard TCP client and communicate with Kickdrive using ASCII messages.

Please refer to the Kickdrive Developer Documentation for details on Kickdrive messages and functions available, and the ASCII message format to use.

Usage:

- In Kickdrive, open the project "demo_automation.kickpro"

- Make sure that the "Remote" module is started (green check mark).

- Open the Docklight script "docklight_demo_automation.pts" using menu Scripting --> Open Script...

- Run the script (Scripting --> Run Script, Shift + F5)

Example communication output could look like this:

blue parts: The requests from Docklight / the TCP client

red parts: The answers from Kickdrive

green parts: additional information generated by Docklight

Launching Kickdrive...

C:\fullmoKickdriveFiles\Kickdrive.exe /uimode=6 "C:\Users\heggelor\Desktop\demo_tcp_server_autostart.kickpro"

Connecting to Kickstart TCP server...

(Make sure Kickstart has the demo_automation.kickpro project open and the 'Remote' module is started / has green mark.)

Connection established. Sending Kickstart commands.

02.07.2020 17:09:58.148 [TX] - CANopen Drive:KM_SETPROPERTY:KS_REQUEST:NodeId=1<LF>

<LF>

02.07.2020 17:09:58.152 [RX] - CANopen Drive:KM_SETPROPERTY:KS_ANSWER:NodeId=1<LF>

<LF>

02.07.2020 17:09:58.161 [TX] - CAN Interface:KM_COMM_CLOSE:KS_REQUEST<LF>

<LF>

02.07.2020 17:09:58.165 [RX] - CAN Interface:KM_COMM_CLOSE:KS_ANSWER CANUSB interface has been closed successfully !

02.07.2020 17:09:58.165 [RX] - <LF>

<LF>

02.07.2020 17:09:58.168 [TX] - CAN Interface:KM_COMM_SET_BAUD:KS_REQUEST:125K<LF>

<LF>

02.07.2020 17:09:58.171 [RX] - CAN Interface:KM_SETPROPERTY:KS_INFO:Baudrate=125K<LF>

<LF>

CAN Interface:KM_COMM_SET_BAUD:KS_ANSWER:125K<LF>

<LF>

02.07.2020 17:09:58.184 [TX] - CAN Interface:KM_COMM_OPEN:KS_REQUEST<LF>

<LF>

02.07.2020 17:09:58.195 [RX] - CAN Interface:KM_INFOTEXT:KS_INFO:Trying usb2drive ver=1 serno= bitrate=125000<LF>

<LF>

CAN Interface:KM_INFOTEXT:KS_INFO:Connected to usb2drive ver=01.01.03.02.Rev2B serno=0000000001<LF>

<LF>

CAN Interface:KM_INFOTEXT:KS_INFO:Version check: CAN V01.01.03.02.Rev2B - ok<LF>

<LF>

CAN Interface:KM_COMM_OPEN:KS_ANSWER CANUSB interface has been opened successfully !

02.07.2020 17:09:59.644 [RX] - <LF>

<LF>

CAN Interface:KM_INFOTEXT:KS_INFO:Open (125K)<LF>

<LF>

02.07.2020 17:09:59.763 [TX] - CANopen Drive:KM_OBJLIST_READ:KS_REQUEST:,100Ah,0h,visible_string<LF>

<LF>

02.07.2020 17:09:59.776 [RX] - CANopen Drive:KM_INFOTEXT:KS_INFO:Start communication<LF>

<LF>

CAN Interface:KM_COMM_OPEN:KS_ANSWER CANUSB interface has been opened successfully !

02.07.2020 17:09:59.776 [RX] - <LF>

<LF>

CANopen Drive:KM_INFOTEXT:KS_INFO:Idle<LF>

<LF>

CANopen Drive:KM_INFOTEXT:KS_INFO:Reading objects ...<LF>

<LF>

CANopen Drive:KM_OBJLIST_READ:KS_ANSWER:,100Ah,0h,visible_string,v1.0.340<LF>

<LF>

CANopen Drive:KM_INFOTEXT:KS_INFO:Reading objects OK<LF>

<LF>

02.07.2020 17:09:59.960 [TX] - CANopen Drive:KM_DRIVE_START:KS_REQUEST<LF>

<LF>

02.07.2020 17:09:59.976 [RX] - CAN Interface:KM_COMM_OPEN:KS_ANSWER CANUSB interface has been opened successfully !

02.07.2020 17:09:59.976 [RX] - <LF>

<LF>

CANopen Drive:KM_INFOTEXT:KS_INFO:Init<LF>

<LF>

CAN Interface_canopen_gateway:KM_CANOPEN_BOOTUP:KS_INFO:1<LF>

<LF>

CANopen Drive:KM_INFOTEXT:KS_INFO:Operating<LF>

<LF>

CANopen Drive:KM_DRIVE_START:KS_ANSWER CANopen Drive has been started successfully !

02.07.2020 17:10:01.559 [RX] - <LF>

<LF>

02.07.2020 17:10:01.676 [TX] - CANopen Drive:KM_DRIVE_STOP:KS_REQUEST<LF>

<LF>

02.07.2020 17:10:01.693 [RX] - CANopen Drive:KM_CANCEL_INTERVIEWS:KS_ANSWER<LF>

<LF>

CANopen Drive:KM_INFOTEXT:KS_INFO:Idle<LF>

<LF>

CANopen Drive:KM_DRIVE_STOP:KS_ANSWER CANopen Drive has been stopped successfully !

02.07.2020 17:10:01.909 [RX] - <LF>

<LF>

Finished drive task, closing Kickdrive host application...

02.07.2020 17:10:02.431 [TX] - :KM_APPLICATION_CONTROL:KS_REQUEST:exit<LF>

<LF>

02.07.2020 17:10:02.549  DOCKLIGHT reports: Error on channel LOCALHOST:51111:

TCP/IP connection closed by the remote computer